robotics

Homework A

Klik en typ de subtitel

Enmin Lin, Kaixi Yao

april 2026

Q1

As Fig. 1 shows, one 3-DOF positioning arm can exhibit one, two, or infinitely inverse position solutions, depending on the target point.

Fig. 1. Three inverse position kinematics cases for a 3-DOF positioning arm with base yaw q1, shoulder pitch q2, and elbow pitch q3.

- In the left panel, the target lies on the workspace boundary. The arm is fully stretched, so the elbow-up and elbow-down branches merge into one singular configuration. This gives one inverse position solution.

- In the middle panel, the target lies at a generic reachable point. The base yaw stays unique, and the planar two-link geometry gives the standard elbow-up and elbow-down configurations. This gives exactly two inverse position solutions.

- In the right panel, the target lies on the base rotation axis. The base yaw does not change the end-effector position in that case. The variable q1 is free, so infinitely many inverse position solutions exist.

Q2

Given initial configuration

qs=(xs,ys,ϕs,θs)

and goal configuration

qg=(xg,yg,ϕg,θg).

Move to the target position (xg,yg).

First turn in place until the heading ϕ points from xsys toward xgyg.

Then roll straight until the contact point reaches xgyg.

This changes x,y, and also changes the rolling angle θ, but that is fine because θ will be corrected later.

Set the desired heading ϕg.

Once the coin is at xgyg, turn in place until ϕ=ϕg.

Turning in place changes the heading but does not change the contact point position.

Correct the rolling angle θ without changing position or heading.

Now compute how much rolling angle is still missing, modulo 2π:

Δθ=θg-θ

modulo 2π.

Execute the following closed maneuver:

- roll forward by a distance L;

- turn in place by π;

- roll forward again by the same distance L;

- turn in place by π.

After this loop, the coin has returned to the same (x,y) and the same heading ϕ. However, it has rolled a total distance 2L, so the rolling angle θ has changed. Choose L so that this change equals the required Δθ modulo 2π.

Therefore, the algorithm reaches the desired final configuration xg,yg,ϕg,θg.

Why this works

The key geometric property is that the non-holonomic constraints forbid sideways instantaneous motion, but they do not remove configurations from the C-space. By combining allowed motions, especially closed loops, we can create net changes in variables that cannot be changed independently at one instant. This is the same idea as parking a car: it cannot move sideways instantly, but it can still reach sideways-displaced configurations through forward/backward and turning maneuvers.

Q3

trR=1+2cosθ

From Q3:

R=00-1-100010

First, calculate the trace:

trR=0

so

0=1+2cosθ,cosθ=-12

Select the principal angle:

θ=2π3

Since sinθ≠0, we can use the matrix logarithm formula:

ω=R-RT2sinθ

Calculating

R-RT=01-1-10-1110

Since

2sin2π3=3

So

ω=1301-1-10-1110

Compare it with the standard skew-symmetric matrix:

ω=0-ω3ω2ω30-ω1-ω2ω10

Then get

ω=131-1-1

Therefore, exponential coordinates are

ωθ=2π3131-1-1=2π33-2π33-2π33

Matrix form is

ωθ=2π3301-1-10-1110

Note: exponential coordinates are not unique, because rotations can be represented by equivalent axis-angles; the above is the principal solution. The pair (-ω,-θ) represents the same rotation, so the equivalent answer also exist.

Q4

Fig. 2. Visibility-graph path for the point robot in the workspace of Homework A.

I implement a complete path planner for a point robot among polygonal obstacles using a visibility graph, following the method from Lecture 5.

The configuration space is the 2D space, since the robot is a point robot. The obstacles are polygonal regions. I chose the start, goal, and obstacle vertices to approximately match the figure in the assignment.

The planner works as follows:

- Add the start point, goal point, and all obstacle vertices as graph nodes.

- For every pair of nodes, check whether the straight-line segment between them intersects any obstacle.

- If the segment is collision-free, add an edge to the visibility graph with Euclidean distance as cost.

- Run A* search on this graph, using Euclidean distance to the goal as the heuristic.

- Return the shortest collision-free path found.

The obstacle vertices used in my implementation are:

start = (2.60, 2.60)

goal  = (7.45, 7.30)

obstacle 1: [(0.45, 5.45), (1.05, 7.35), (3.45, 7.25), (4.0, 5.0), (2.05, 6.45)]

obstacle 2: [(5.25, 5.15), (6.05, 6.85), (7.3, 4.75)]

obstacle 3: [(2.05, 3.75), (4.05, 4.2), (3.25, 3.2), (3.75, 2.0)]

The computed shortest path is:

(2.60, 2.60)

(2.05, 3.75)

(4.00, 5.00)

(6.05, 6.85)

(7.45, 7.30)

With total length: 7.822886

This planner is complete for this setting because, for a point robot with polygonal obstacles, a shortest collision-free path consists of straight-line segments whose turning points occur at obstacle vertices. Therefore, the visibility graph contains the optimal path candidate, and A* finds the shortest path in that graph.